EXP9 PERFORMANCE EVALUATION OF AM GENERATION AND DETECTION UNDER NOISY CHANNEL CONDITIONS USING MATLAB

Objectives:

i) To generate an Amplitude Modulated (AM) signal, introduce Additive White Gaussian Noise

(AWGN), and analyze the effect of noise on the transmitted signal using MATLAB.

ii) To recover the original message signal from the noisy AM signal using envelope detection

and compare the transmitted and recovered waveforms to evaluate the performance of the AM

communication system.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fm = 500;

fc = 5000;

% Message Signal

m = cos(2*pi*fm*t);

c = cos(2*pi*fc*t);

% AM Modulation

am = (1 + m).*cos(2*pi*fc*t);

% Add AWGN Noise

noisy = awgn(am,10,'measured'); % 10 dB SNR

% Envelope Detection

env = abs(hilbert(noisy));

% Display Results

figure;

subplot(5,1,1)

plot(t,m)

title('Message Signal')

grid on

subplot(5,1,2)

plot(t,c)

title('Carrier Signal')

grid on

subplot(5,1,3)

plot(t,am)

title('AM Signal')

grid on

subplot(5,1,4)

plot(t,noisy)

title('AM Signal with AWGN Noise')

grid on

subplot(5,1,5)

plot(t,env)

title('Recovered Signal')

grid on

Procedure:

1. Open MATLAB and create a new script file.

2. Define the simulation parameters, including the sampling frequency, time vector, message

frequency, carrier frequency, message amplitude, carrier amplitude, and Signal-to-Noise Ratio

(SNR).

3. Generate the message signal using a low-frequency sinusoidal waveform.

4. Generate the carrier signal using a high-frequency sinusoidal waveform.

5. Generate the Amplitude Modulated (AM) signal by modulating the carrier signal with the

message signal.

6. Introduce Additive White Gaussian Noise (AWGN) to the AM signal using the MATLAB

awgn() function with the specified SNR.

7. Perform envelope detection on the noisy AM signal using the Hilbert transform to recover

the original message signal.

8. Plot the message signal, carrier signal, AM signal, noisy AM signal, and recovered signal

using MATLAB subplots.

9. Compare the original message signal with the recovered signal to observe the effect of

AWGN on signal quality.

10. Analyze the performance of the AM communication system by studying the degradation

caused by noise and the effectiveness of the envelope detector in recovering the message signal.

11. Verify the successful generation, transmission, and recovery of the AM signal under noisy

channel conditions.

OUTPUT:

RESULT:

i) The AM signal was successfully generated in MATLAB, and the effect of Additive White Gaussian Noise (AWGN) on the transmitted signal was observed through the noisy AM waveform. ii) The original message signal was successfully recovered using envelope detection, demonstrating the effectiveness of the AM receiver in recovering the transmitted information in the presence of AWGN noise.